SOFTWARE REQUIREMENTS SPECIFICATION

Jessica — Solar-Powered Agricultural Robot System

Document ID: JES-SRS-001

Version: 1.0

Date: 7.11.25

Project: Jessica-HIT

Repository: Jessica-HIT (monorepo)

Prepared by: Lidor Elmakaies, Amit Leiba, Matan Heinrich

# Table of Contents

1  Introduction

1.1  Purpose

1.2  Scope

1.3  Definitions, Acronyms, and Abbreviations

1.4  Document Conventions

1.5  Intended Audience

2  Overall Description

2.1  Product Perspective

2.2  Product Functions — Summary

2.3  User Classes and Characteristics

2.4  Operating Environment

2.5  Design and Implementation Constraints

2.6  Assumptions and Dependencies

3  System Architecture

3.1  End-to-End Communication Chain

3.2  Subsystem Overview

4  Embedded Firmware — STM32 Robot

4.1  Hardware Platform

4.2  Firmware Architecture

4.3  Serial Protocol (UART)

4.4  Driver Functional Requirements

5  Embedded Firmware — ESP32 Gateway

5.1  Hardware Platform

5.2  BLE Client

5.3  WebSocket Server

5.4  Message Routing

6  Backend Services

6.1  Gateway Service

6.2  Authentication Service

6.3  JessicaManager Service

6.4  Recording Manager Service

6.5  Metrics Service

7  Frontend Application

7.1  Feature Modules

7.2  State Management

7.3  Core Services

8  Robot Simulator

9  External Interface Requirements

9.1  Hardware Interfaces

9.2  Software Interfaces

9.3  Communication Interfaces

10  Non-Functional Requirements

11  Security Requirements

12  Appendix A — API Endpoint Reference

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) describes the complete functional and non-functional requirements of the Jessica system — a solar-powered agricultural robot with remote control and monitoring capabilities. The document covers all system layers: embedded firmware running on the robot, a Bluetooth-to-WebSocket gateway, and a full-stack web application for remote operation, telemetry visualization, session recording, and metrics analytics.

This document is intended for use by the development team, hardware integrators, and academic stakeholders at HIT (Holon Institute of Technology).

## 1.2 Scope

The Jessica-HIT system provides the following capabilities:

Real-time remote control of a physical agricultural robot over the internet.

Live telemetry streaming: obstacle distance, safety state, drive mode, and battery voltage.

Autonomous obstacle-detection safety enforcement on the embedded firmware.

Session recording and playback of movement sequences.

Historical metrics collection and visualization with export (CSV, PDF).

Role-based access control for multi-user operation.

A Python-based robot simulator for development and testing without hardware.

## 1.3 Definitions, Acronyms, and Abbreviations

| Term / Acronym | Definition |
| --- | --- |
| SRS | Software Requirements Specification |
| MCU | Microcontroller Unit |
| UART | Universal Asynchronous Receiver-Transmitter |
| DMA | Direct Memory Access |
| BLE | Bluetooth Low Energy |
| ESP32 | Espressif ESP32 dual-core Wi-Fi and Bluetooth MCU |
| STM32 | STMicroelectronics STM32F103C8 ARM Cortex-M3 MCU ("Blue Pill") |
| YARP | Yet Another Reverse Proxy — Microsoft reverse proxy library |
| SignalR | ASP.NET Core real-time communication library over WebSocket/SSE |
| JWT | JSON Web Token — compact, URL-safe token standard (RFC 7519) |
| NgRx | Reactive state management library for Angular based on Redux |
| FSM | Finite State Machine |
| HC-SR04 | Ultrasonic distance sensor (2 cm – 4 m range) |
| HM-10 | Bluetooth 4.0 BLE UART module attached to STM32 via UART |
| PWM | Pulse Width Modulation — used for motor speed control |
| H-Bridge | Motor driver circuit enabling bidirectional motor control |
| REST | Representational State Transfer — HTTP API architectural style |
| SPA | Single Page Application |
| ORM | Object Relational Mapper |
| EF Core | Entity Framework Core — .NET ORM for database access |
| PlatformIO | Open-source IDE and build system for embedded development |
| Aspire | .NET Aspire — local microservice orchestration framework |

## 1.4 Document Conventions

Requirements are stated using "shall" (mandatory), "should" (recommended), "may" (optional).

Code snippets and protocol frames are shown in monospace font.

Requirement identifiers follow the pattern FR-<subsystem>-<number> and NFR-<number>.

## 1.5 Intended Audience

| Audience | Relevant Sections |
| --- | --- |
| Embedded firmware developers | 4, 5, 9.1, 9.3 |
| Backend / API developers | 6, 9.2, 11, 12 |
| Frontend developers | 7, 9.2 |
| System architects / academic supervisors | 1, 2, 3, 10 |
| DevOps / deployment engineers | 2.4, 6, 10 |
| QA / testers | 8, 10, 12 |

# 2. Overall Description

## 2.1 Product Perspective

Jessica is a standalone embedded-plus-cloud system. It does not depend on any external cloud platform. All backend services are self-hosted. The robot hardware communicates wirelessly through a two-hop chain: a wired UART link to the ESP32 gateway, then a BLE link (or direct UART-over-BLE), then a WiFi/WebSocket link to the backend, and finally SignalR to the browser-based frontend.

## 2.2 Product Functions — Summary

Robot Control: send directional move and stop commands in real time.

Telemetry Streaming: receive 4 Hz status updates (distance, safety, mode, battery) from the robot.

Safety Supervision: automatic motor cut-off when an obstacle is detected within 15 cm.

Session Recording & Playback: record movement sequences and replay them.

Metrics Analytics: persist and visualize historical sensor readings with aggregated statistics.

Live Feed: display a live camera stream from the robot.

User Management: role-based account creation and administration.

Robot Simulator: software-only robot for development without physical hardware.

## 2.3 User Classes and Characteristics

| Role | Permissions | Description |
| --- | --- | --- |
| Operator | Control robot, view telemetry, manage own recordings, view metrics, view live feed | Primary robot operator. Expected to have basic technical training. |
| Admin | All Operator permissions + manage users (create, delete, assign roles), clear all metrics | System administrator. Can manage user accounts. |
| Viewer (default) | View public info, authenticate | Newly registered account before role assignment. |

## 2.4 Operating Environment

Robot firmware: STM32F103C8 running bare-metal C, no RTOS, 72 MHz ARM Cortex-M3.

Gateway firmware: ESP32 running Arduino framework via PlatformIO, connected to WiFi 2.4 GHz.

Backend: .NET 9 microservices, containerised with Docker Compose, PostgreSQL 15+.

Frontend: Angular 18 SPA, served via Nginx in Docker or ng serve for development.

Supported browsers: Chromium-based (Chrome, Edge), Firefox, Safari (modern versions).

Development orchestration: .NET Aspire (local), Docker Compose (staging/production).

## 2.5 Design and Implementation Constraints

The STM32 has 64 KB Flash and 20 KB RAM — no dynamic memory allocation, no RTOS.

ESP32 BLE client connects by fixed MAC address; the STM32 BLE module MAC must not change.

WebSocket server on ESP32 listens on port 81; the backend must connect to this exact port.

JWT shared secret must be consistent across all backend services (set via environment variable).

All backend services must run on .NET 9.0 with nullable reference types enabled.

Robot telemetry frame format is fixed: $STATUS,<dist>,<safety>,<mode>,<battery_mv>.

## 2.6 Assumptions and Dependencies

The robot WiFi network (SSID/password) is configured before flashing the ESP32 firmware.

The ESP32 BLE MAC address of the STM32 module is known at compile time.

The backend server is reachable from the ESP32 over the same WiFi network or the internet.

PostgreSQL instances are provisioned and accessible before backend services start.

The HC-SR04 sensor is correctly wired to the STM32 EXTI pins.

PlatformIO CLI is installed for ESP32 firmware builds.

# 3. System Architecture

## 3.1 End-to-End Communication Chain

The full communication path from the physical robot to the operator browser:

[STM32 Robot] ──UART──> [ESP32 Gateway] ──BLE──> [HM-10 Module]
         └──────WiFi/WebSocket (port 81)──────> [JessicaManager]
                                                      │
                                               [Gateway :5207]
                                               SignalR + REST
                                                      │
                                          [Angular Frontend :4200]

Protocol summary per segment:

| Segment | Protocol | Format |
| --- | --- | --- |
| STM32 → ESP32 (wired) | UART | $STATUS,<dist>,<safety>,<mode>,<battery_mv> |
| STM32 ↔ BLE Module | Bluetooth LE (service FFE0, char FFE1) | $STATUS… / $M,<l>,<r> |
| ESP32 ↔ JessicaManager | WebSocket (port 81) | JSON |
| Frontend ↔ Gateway | SignalR (WebSocket) + REST over HTTPS | JSON / JWT Bearer |
| Gateway → Microservices | HTTP (YARP reverse proxy) | JSON / JWT |

## 3.2 Subsystem Overview

| Subsystem | Technology | Primary Responsibility |
| --- | --- | --- |
| Robot Firmware (STM32) | C, Keil MDK-ARM, STM32 SPL | Motor control, sensor reading, safety enforcement, telemetry TX |
| Gateway Firmware (ESP32) | C++, Arduino, PlatformIO | BLE↔WebSocket bridging, protocol translation |
| Gateway Service | .NET 9, YARP, SignalR, ASP.NET Core | API entry point, routing, auth, real-time relay |
| AuthService | .NET 9, EF Core, PostgreSQL, BCrypt | User accounts, JWT issuance, role management |
| JessicaManager | .NET 9, WebSocket client | Robot WebSocket bridge, command dispatch, status cache |
| RecordingManager | .NET 9, EF Core, PostgreSQL | Movement recording CRUD and replay data |
| MetricsService | .NET 9, EF Core, PostgreSQL | Sensor data collection, query, statistics |
| Angular Frontend | Angular 18, NgRx, PrimeNG, SignalR | Operator UI — control, monitoring, analytics |
| Robot Simulator | Python 3, websockets | Software robot for development/testing |

# 4. Embedded Firmware — STM32 Robot

## 4.1 Hardware Platform

| Item | Specification |
| --- | --- |
| MCU | STM32F103C8 ("Blue Pill") |
| CPU Core | ARM Cortex-M3 @ 72 MHz |
| Flash | 64 KB (0x08000000) |
| RAM | 20 KB (0x20000000) |
| Toolchain | Keil MDK-ARM v5, ARM Compiler v5.06 |
| Distance Sensor | HC-SR04 (2 cm – 4 m, EXTI-based non-blocking) |
| Motor Driver | Dual H-bridge (left/right PWM via TIM channels) |
| Communication | HM-10 BLE UART module (UART + DMA ring buffer) |
| LED Strip | WS2812B RGB (NeoPixel compatible) |
| Line Sensor | IR line tracking array |
| Servo | Single servo motor |
| Power | Solar-charged battery (ADC battery voltage monitoring) |

## 4.2 Firmware Architecture

The firmware implements a non-blocking FSM super-loop driven by a 1 ms SysTick timer. There is no RTOS. All peripheral drivers use non-blocking patterns (DMA ring buffers, EXTI interrupts) to avoid stalling the main loop.

Main loop execution sequence (every iteration):

Trigger HC-SR04 ultrasonic measurement every 100 ms.

Run Safety_Task_Update() to evaluate obstacle proximity.

Parse any queued UART command from the DMA ring buffer.

Transmit telemetry $STATUS frame every 250 ms (4 Hz).

## 4.3 Serial Protocol (UART)

Telemetry frame (STM32 → ESP32), 4 Hz:

$STATUS,<distance_cm>,<safety_state>,<mode>,<battery_mv>

| Field | Type | Description |
| --- | --- | --- |
| distance_cm | Integer | HC-SR04 reading in centimetres |
| safety_state | Integer | 0 = CLEAR, 1 = WARNING, 2 = CRITICAL STOP |
| mode | Integer | Current drive mode (see mode table below) |
| battery_mv | Integer | Battery voltage in millivolts (from ADC) |

Drive mode values:

| Value | Mode |
| --- | --- |
| 0 | IDLE |
| 1 | MOVING |
| 2 | AUTONOMOUS |
| 3 | CHARGING |

Command frames (ESP32 → STM32):

| Command | Frame Format | Description |
| --- | --- | --- |
| Move | $M,<left>,<right>\n | Set left/right motor speeds (signed integer) |
| Stop | $S\n | Emergency stop — zero both motors immediately |

## 4.4 Driver Functional Requirements

| Requirement ID | Requirement |
| --- | --- |
| FR-STM-01 | UART_DMA shall receive command frames using DMA into a ring buffer without blocking the main loop. |
| FR-STM-02 | UART_ProcessCommand() shall parse $M and $S frames and apply motor values only when SafetyState is not CRITICAL. |
| FR-STM-03 | UART_SendTelemetry() shall transmit a $STATUS frame at 4 Hz (every 250 ms). |
| FR-STM-04 | Ultrasonic_Trigger() shall fire the HC-SR04 trigger pulse every 100 ms; echo timestamps shall be captured in ISR. |
| FR-STM-05 | Safety_Task_Update() shall return SAFETY_CRITICAL when distance < 15 cm and forcibly cut PWM output. |
| FR-STM-06 | Battery_Read_Voltage() shall read the ADC and convert the raw value to millivolts. |
| FR-STM-07 | PWM_Motor shall support signed speed values (negative = reverse) for each motor independently. |

# 5. Embedded Firmware — ESP32 Gateway

## 5.1 Hardware Platform

| Item | Specification |
| --- | --- |
| MCU | ESP32 (generic dev board) |
| Framework | Arduino via PlatformIO (espressif32) |
| Flash Partition | huge_app.csv |
| Upload Port | COM3 (configurable in platformio.ini) |
| Monitor Baud Rate | 115200 |
| Key Libraries | ArduinoJson 6.21.3, WebSockets 2.4.1 |

## 5.2 BLE Client

The ESP32 acts as a BLE central (client). It connects to the STM32 robot by fixed MAC address using the HM-10 BLE UART service:

Service UUID: FFE0
Characteristic UUID: FFE1
Default MAC: F8:33:31:FC:70:BB

FR-ESP-01: The BLE client shall connect to the STM32 module on startup and register a notification callback on characteristic FFE1.

FR-ESP-02: The BLE client shall attempt to reconnect automatically if the connection is lost.

## 5.3 WebSocket Server

The ESP32 hosts a WebSocket server on port 81 that accepts connections from the JessicaManager backend service.

WebSocketsServer webSocket(81);  // listens on all interfaces

FR-ESP-03: The WebSocket server shall support multiple simultaneous connected clients; telemetry shall be broadcast to all.

## 5.4 Message Routing

| Direction | Source Format | Destination Format |
| --- | --- | --- |
| Telemetry: BLE → WebSocket | $STATUS,<dist>,<safety>,<mode>,<battery_mv> | {"type":"telemetry","distance":int,"safety":int,"mode":int,"battery":float} |
| Command: WebSocket → BLE | {"cmd":"move","left":int,"right":int} | $M,<left>,<right>\n |
| Stop: WebSocket → BLE | {"cmd":"stop"} | $S\n |

FR-ESP-04: The gateway shall translate every BLE notification into a JSON telemetry message and broadcast it over WebSocket within 50 ms.

FR-ESP-05: The gateway shall translate every incoming WebSocket JSON command into the corresponding UART-over-BLE frame within 50 ms.

# 6. Backend Services

All backend services are .NET 9.0 microservices with nullable reference types enabled. JWT validation is performed by each service independently using a shared secret.

| Service | Default Port | Database | Purpose |
| --- | --- | --- | --- |
| Gateway | 5207 (HTTP) / 7215 (HTTPS) | None | YARP reverse proxy, SignalR hub, JWT validation, rate limiting |
| AuthService | 5100 | jessica_auth (port 5433) | Login, register, JWT issuance, user CRUD, role management |
| JessicaManager | 5000 (HTTP) / 5001 (HTTPS) | None (in-memory state) | WebSocket robot bridge, command dispatch, status cache |
| RecordingManager | 5300 | jessica_recordings (port 5434) | Recording session CRUD and event persistence |
| MetricsService | 5080 | jessica_metrics (port 5435) | Sensor data ingestion, query, and statistics |

## 6.1 Gateway Service

FR-GW-01: The Gateway shall validate JWT Bearer tokens on all protected routes before forwarding to upstream services.

FR-GW-02: The Gateway shall apply a rate limit of 10 requests per 10 seconds to all authentication endpoints.

FR-GW-03: The Gateway shall host a SignalR hub at /hubs/jessica and relay robot status events pushed by JessicaStatusRelayService.

FR-GW-04: The Gateway shall forward the authenticated user's ID to upstream services via the X-User-Id request header transform.

FR-GW-05: The Gateway shall route all /api/* requests to the appropriate microservice via YARP configuration.

## 6.2 Authentication Service

FR-AUTH-01: The service shall hash all passwords using BCrypt before persistence.

FR-AUTH-02: The service shall issue HS256 JWT access tokens and refresh tokens on successful login.

FR-AUTH-03: The service shall support token rotation via the /api/auth/refresh endpoint.

FR-AUTH-04: Admin users shall be able to create users with explicit role assignments.

FR-AUTH-05: Admin users shall not be able to delete or change the role of their own account.

FR-AUTH-06: Passwords shall be at least 8 characters long.

Authentication endpoints:

| Method | Path | Auth Required | Description |
| --- | --- | --- | --- |
| POST | /api/auth/login | None | Login with username + password; returns JWT access + refresh tokens |
| POST | /api/auth/register | None | Self-register (assigned Viewer role by default) |
| POST | /api/auth/refresh | None | Rotate access/refresh token pair |
| GET | /api/auth/user-info | JWT | Return current user info and claims |
| GET | /api/auth/logout | JWT | Revoke all refresh tokens for the current user |
| GET | /api/auth/roles | JWT | Return roles extracted from current token |
| GET | /api/Auth/Users | JWT + Admin | List all users with their roles |
| POST | /api/Auth/Users | JWT + Admin | Admin creates a user and assigns role |
| DELETE | /api/Auth/Users/{id} | JWT + Admin | Delete a user (cannot self-delete) |
| PUT | /api/Auth/Users/{id}/role | JWT + Admin | Update a user's role (cannot self-change) |
| GET | /api/health | None | Health check endpoint |

## 6.3 JessicaManager Service

FR-JM-01: JessicaManager shall maintain a persistent WebSocket client connection to the robot (or simulator).

FR-JM-02: The service shall accept HTTP POST commands and translate them to JSON WebSocket messages sent to the robot.

FR-JM-03: The service shall cache the most recent telemetry state received from the robot and serve it via GET /api/car/status.

FR-JM-04: If no telemetry has been received in the last 2 seconds, /api/car/status shall return available: false.

FR-JM-05: The robot IP/URL shall be persisted to gateway-ip.json and survive service restarts.

FR-JM-06: The service shall support direction-to-motor-value conversion (forward, backward, left, right, stop).

JessicaManager endpoints:

| Method | Path | Auth | Description |
| --- | --- | --- | --- |
| POST | /api/car/direction | X-User-Id | Move robot in a direction at a given speed |
| POST | /api/car/speed | X-User-Id | Set movement speed without changing direction |
| POST | /api/car/start | X-User-Id | Start a control session |
| POST | /api/car/stop | X-User-Id | Emergency stop — zero both motors |
| GET | /api/car/status | X-User-Id | Poll latest robot telemetry (stale if > 2 s) |
| PUT | /api/connection/gateway-ip | X-User-Id | Update the ESP32 WebSocket URL |
| GET | /api/connection/status | None | Diagnostic: test current WebSocket connection (4 s timeout) |
| POST | /api/connection/refresh | X-User-Id | Force immediate WebSocket reconnect to robot |

Robot status response fields:

| Field | Type | Description |
| --- | --- | --- |
| available | Boolean | False if no telemetry received in last 2 seconds |
| distance | Integer | Obstacle distance in centimetres |
| safety | Integer | 0=CLEAR, 1=WARNING, 2=CRITICAL |
| mode | Integer | 0=IDLE, 1=MOVING, 2=AUTONOMOUS, 3=CHARGING |
| battery | Float | Battery voltage (3.0 – 4.2 V) |
| receivedAtUtc | DateTime | UTC timestamp of last telemetry frame |

## 6.4 Recording Manager Service

FR-REC-01: The service shall store recording sessions per user; users may only access their own recordings.

FR-REC-02: A recording shall capture: name (max 200 chars), speed (0–100), total duration in ms, and an ordered list of movement events.

FR-REC-03: Each event shall store: time offset in ms from recording start and direction string (max 50 chars).

FR-REC-04: The service shall support create, read (summary list), read (detail with events), and delete operations.

Recording endpoints:

| Method | Path | Auth | Description |
| --- | --- | --- | --- |
| GET | /api/recordings | X-User-Id | List current user's recordings (summaries only) |
| GET | /api/recordings/{id} | X-User-Id | Get recording detail with full event list for replay |
| POST | /api/recordings | X-User-Id | Create a new recording (201 Created) |
| DELETE | /api/recordings/{id} | X-User-Id | Delete recording and all its events (204 No Content) |

## 6.5 Metrics Service

FR-MET-01: A background worker (MetricsCollectorWorker) shall periodically poll JessicaManager and persist the latest telemetry as a timestamped metric record.

FR-MET-02: The /api/metrics/history endpoint shall return timestamped records for a requested duration (default 60 minutes).

FR-MET-03: If query results exceed 500 data points, the service shall downsample to approximately 500 points for frontend chart performance.

FR-MET-04: The /api/metrics/stats endpoint shall return aggregate statistics: counts, averages, min/max, safety incidents, and mode distribution.

FR-MET-05: Only Admin users may delete all metrics via DELETE /api/metrics/all.

Metrics endpoints:

| Method | Path | Auth | Description |
| --- | --- | --- | --- |
| GET | /api/metrics/history | JWT | Historical sensor readings (durationMinutes param, default 60) |
| GET | /api/metrics/stats | JWT | Aggregated statistics for a time window |
| DELETE | /api/metrics/all | JWT + Admin | Clear all metrics records |

Supported time range query values from the frontend: 15, 60, 360, 1440 minutes (15 min, 1 h, 6 h, 24 h).

# 7. Frontend Application

The Angular 18 SPA communicates with the backend via the Gateway Service (SignalR for real-time events, REST for commands and queries). All HTTP requests carry a JWT Bearer token injected by the auth interceptor.

## 7.1 Feature Modules

| Module | Route | Access | Description |
| --- | --- | --- | --- |
| Auth | /auth | Public | Login and self-registration screen (dual-mode panel) |
| Home (Dashboard) | /home | Authenticated | System status overview, connection diagnostics, current robot state |
| Manual Controller | /manual-controller | Operator+ | D-pad directional control, speed dial, real-time sensor data panel |
| Live Feed | /live-feed | Operator+ | Camera stream display (URL configured via environment.cameraUrl) |
| Recorder | /recorder | Operator+ | Three-tab panel: record new session, replay existing, delete recordings |
| Metrics | /metrics | Operator+ | Historical charts (voltage, distance), stats panel, PDF/CSV export, 3 s auto-refresh |
| User Management | /user-management | Admin | Create, update role, and delete user accounts via modal dialogs |

## 7.2 State Management

The frontend uses NgRx 18 with the following store slices:

auth — current user, JWT token, authentication status.

car — latest robot telemetry state, connection status, current speed.

recording — recording list, active recording detail, playback state.

NgRx Effects handle all asynchronous operations: API calls, SignalR event subscriptions, and token refresh flows.

## 7.3 Core Services

| Service | Responsibility |
| --- | --- |
| auth.service.ts | Login/register API calls, JWT storage (localStorage), token expiry check |
| connection.service.ts | SignalR hub lifecycle: connect, disconnect, reconnect |
| recording.service.ts | RecordingManager API calls (list, get, create, delete) |
| metrics.service.ts | MetricsService API calls (history, stats) |
| auth.interceptor.ts | Injects Authorization: Bearer <token> on every outgoing HTTP request |
| auth.guard.ts | Redirects unauthenticated users to /auth |
| role.guard.ts | Restricts route access based on user role claims |
| SignalManagerService | Subscribes to robot status events from Gateway SignalR hub |

# 8. Robot Simulator

The Python-based robot simulator (jessica-simulator/app.py) provides a software-only replacement for the physical robot during development and testing. It implements the same WebSocket protocol as the ESP32 gateway.

FR-SIM-01: The simulator shall listen on a configurable host and port (default: 127.0.0.1:8765).

FR-SIM-02: The simulator shall accept {"cmd":"move","left":int,"right":int} and {"cmd":"stop"} commands.

FR-SIM-03: The simulator shall emit random but realistic telemetry at a configurable interval (default 1.0 second).

FR-SIM-04: Generated telemetry shall include: distance (0–999 cm), safety (0–2), mode (0–3), battery (3.0–4.2 V).

FR-SIM-05: The simulator shall support verbose logging via --verbose flag.

Usage:

cd Web/jessica-simulator
pip install -r requirements.txt
python app.py --host 127.0.0.1 --port 8765 --telemetry-interval 1.0

To connect JessicaManager to the simulator, set ConnectionSettings:RobotWebSocketUrl = ws://localhost:8765 in appsettings.json or as an environment variable.

# 9. External Interface Requirements

## 9.1 Hardware Interfaces

| Interface | Hardware | Description |
| --- | --- | --- |
| UART (STM32 ↔ HM-10) | STM32 USART peripheral + DMA | Command RX / telemetry TX at configured baud rate; DMA ring buffer |
| HC-SR04 | STM32 GPIO + EXTI | Trigger pin (output), Echo pin (EXTI interrupt input) |
| H-Bridge Motors | STM32 TIM PWM channels | Left and right motor PWM with direction GPIO |
| Battery ADC | STM32 ADC channel | Voltage divider input → millivolt conversion |
| WS2812B LED | STM32 GPIO bitbang | RGB LED status strip |
| Servo | STM32 TIM PWM | Servo position control |
| BLE (ESP32 ↔ HM-10) | ESP32 BLE stack | BLE central scanning and connection to FFE0/FFE1 service |
| WiFi (ESP32) | ESP32 WiFi stack | 2.4 GHz WiFi client connecting to LAN/internet |

## 9.2 Software Interfaces

| Interface | Technology | Description |
| --- | --- | --- |
| Frontend ↔ Gateway | Angular HttpClient + @microsoft/signalr | REST API calls + SignalR real-time events |
| Gateway ↔ Microservices | YARP HTTP proxy | Forwarded HTTP requests with header transforms |
| Microservices ↔ PostgreSQL | EF Core 9 + Npgsql | ORM-managed schema and queries |
| JessicaManager ↔ Robot | System.Net.WebSockets | Persistent WebSocket client connection |
| ESP32 ↔ Backend | WebSockets 2.4.1 (Arduino) | WebSocket client on ESP32 connecting to JessicaManager |

## 9.3 Communication Interfaces

| Interface | Port | Notes |
| --- | --- | --- |
| ESP32 WebSocket server | 81 | Listens for JessicaManager connection |
| Robot Simulator WebSocket | 8765 | Listens for JessicaManager connection (dev only) |
| Gateway HTTP | 5207 | All external API traffic entry point |
| Gateway HTTPS | 7215 | TLS entry point |
| SignalR Hub | /hubs/jessica via port 5207 | Real-time robot status relay to frontend |
| AuthService | 5100 | Internal — reached via Gateway YARP |
| JessicaManager | 5000 / 5001 | Internal — reached via Gateway YARP |
| RecordingManager | 5300 | Internal — reached via Gateway YARP |
| MetricsService | 5080 | Internal — reached via Gateway YARP |
| PostgreSQL (auth) | 5433 | Internal database |
| PostgreSQL (recordings) | 5434 | Internal database |
| PostgreSQL (metrics) | 5435 | Internal database |
| Aspire Dashboard | 15888 | Development only — service health and logs |

# 10. Non-Functional Requirements

| ID | Category | Requirement |
| --- | --- | --- |
| NFR-01 | Performance | Robot telemetry shall be delivered end-to-end (STM32 → browser) in under 500 ms under normal network conditions. |
| NFR-02 | Performance | The metrics history endpoint shall return results within 2 seconds for queries spanning up to 24 hours. |
| NFR-03 | Performance | The Angular frontend shall achieve a Lighthouse performance score ≥ 80 on the dashboard page. |
| NFR-04 | Reliability | The ESP32 gateway shall attempt automatic BLE reconnection within 5 seconds of disconnection. |
| NFR-05 | Reliability | JessicaManager shall mark robot status as unavailable after 2 seconds of no telemetry, preventing stale command dispatch. |
| NFR-06 | Safety | The STM32 safety supervisor shall cut motor PWM within one main-loop iteration (≤ 2 ms) of detecting a CRITICAL obstacle state. |
| NFR-07 | Safety | Motor commands shall be ignored at the firmware level when SafetyState == CRITICAL, regardless of the origin. |
| NFR-08 | Scalability | The WebSocket server on the ESP32 shall support at least 3 simultaneous connected clients. |
| NFR-09 | Maintainability | All backend services shall expose a /api/health endpoint returning HTTP 200 when healthy. |
| NFR-10 | Maintainability | All backend services shall be containerisable via Docker and orchestratable via Docker Compose. |
| NFR-11 | Usability | The manual controller shall respond to directional input within 100 ms as observed in the browser. |
| NFR-12 | Portability | The backend shall run on any platform supporting .NET 9 runtime (Linux, Windows, macOS). |
| NFR-13 | Testability | Backend services shall maintain unit-test coverage for all business logic layers (Domain + Application). |
| NFR-14 | Observability | All backend services shall emit structured telemetry (logs, traces, metrics) compatible with .NET Aspire and OpenTelemetry. |

# 11. Security Requirements

| ID | Requirement |
| --- | --- |
| SEC-01 | All user passwords shall be hashed using BCrypt before storage; plaintext passwords shall never be persisted. |
| SEC-02 | JWT access tokens shall be signed using HMAC-SHA256 (HS256) with a configurable secret key of at least 32 characters. |
| SEC-03 | The JWT secret key shall be supplied via environment variable (Jwt__SecretKey) and shall not be committed to source control. |
| SEC-04 | All inter-service communication shall validate the JWT on every request at the service boundary. |
| SEC-05 | Authentication endpoints shall be rate-limited to 10 requests per 10 seconds to mitigate brute-force attacks. |
| SEC-06 | Admin-only endpoints shall enforce role claim validation in addition to JWT validation. |
| SEC-07 | Admin users shall be prohibited from deleting or modifying the role of their own account. |
| SEC-08 | Recording and metrics data shall be isolated per user via the X-User-Id header; cross-user data access shall be denied. |
| SEC-09 | The frontend shall store the JWT only in localStorage; the application shall clear it on logout. |
| SEC-10 | WebSocket connections from the ESP32 to the backend shall operate over a network trusted by the deployment environment; TLS termination is recommended for production. |
| SEC-11 | WiFi credentials and BLE MAC addresses in the ESP32 firmware shall be treated as sensitive configuration and must be updated before deploying to production networks. |

# 12. Appendix A — Complete API Endpoint Reference

All endpoints below are accessed through the Gateway Service (default: http://localhost:5207). The Gateway adds X-User-Id to forwarded requests from the JWT claim. "JWT" = JWT Bearer token required. "Admin" = Admin role required.

## Authentication Endpoints (/api/auth)

| Method | Path | Auth | Request Body | Response |
| --- | --- | --- | --- | --- |
| POST | /api/auth/login | None | {"username":"str","password":"str"} | {"accessToken":"str","refreshToken":"str","userInfo":{...}} |
| POST | /api/auth/register | None | {"username":"str","email":"str","firstName":"str","lastName":"str","password":"str (min 8)"} | {"userId":"guid","message":"str"} |
| POST | /api/auth/refresh | None | {"accessToken":"str","refreshToken":"str"} | {"accessToken":"str","refreshToken":"str","userInfo":{...}} |
| GET | /api/auth/user-info | JWT | — | {"isAuthenticated":bool,"username":"str","claims":[...]} |
| GET | /api/auth/logout | JWT | — | {"message":"str"} |
| GET | /api/auth/roles | JWT | — | {"Roles":["str"]} |
| GET | /api/Auth/Users | JWT+Admin | — | [{"id":"guid","username":"str","email":"str","role":"str",...}] |
| POST | /api/Auth/Users | JWT+Admin | {"username":"str","email":"str","firstName":"str","lastName":"str","password":"str","role":"str"} | {"userId":"guid","message":"str"} |
| DELETE | /api/Auth/Users/{id} | JWT+Admin | — | {"message":"User deleted successfully"} |
| PUT | /api/Auth/Users/{id}/role | JWT+Admin | {"role":"str"} | {"message":"User role updated successfully"} |

## Robot Command Endpoints (/api/car, /api/connection)

| Method | Path | Auth | Request Body | Response |
| --- | --- | --- | --- | --- |
| POST | /api/car/direction | JWT | {"connectionId":"str","direction":"str","speed":int} | {"accepted":true,"direction":"str","speed":int,"leftMotor":int,"rightMotor":int} |
| POST | /api/car/speed | JWT | {"connectionId":"str","speed":int} | {"accepted":true,"speed":int} |
| POST | /api/car/start | JWT | {"connectionId":"str"} | {"accepted":true} |
| POST | /api/car/stop | JWT | {"connectionId":"str"} | {"accepted":true,"leftMotor":0,"rightMotor":0} |
| GET | /api/car/status | JWT | — | {"available":bool,"distance":int,"safety":int,"mode":int,"battery":float,"receivedAtUtc":"datetime"} |
| PUT | /api/connection/gateway-ip | JWT | {"url":"str"} | {"success":true,"url":"str"} |
| GET | /api/connection/status | None | — | {"targetUrl":"str","reachable":bool,"error":"str¦null"} |
| POST | /api/connection/refresh | JWT | — | {"success":true} |

## Recording Endpoints (/api/recordings)

| Method | Path | Auth | Request Body | Response |
| --- | --- | --- | --- | --- |
| GET | /api/recordings | JWT | — | [{"id":"guid","name":"str","speed":int,"durationMs":long,"createdAt":"datetime"}] |
| GET | /api/recordings/{id} | JWT | — | {"id":"guid","name":"str","speed":int,"durationMs":long,"createdAt":"datetime","events":[{"offsetMs":long,"direction":"str"}]} |
| POST | /api/recordings | JWT | {"name":"str (max 200)","speed":int (0-100),"durationMs":long,"events":[{"offsetMs":long,"direction":"str (max 50)"}]} | RecordingDetailResponse (201 Created) |
| DELETE | /api/recordings/{id} | JWT | — | 204 No Content |

## Metrics Endpoints (/api/metrics)

| Method | Path | Auth | Query Params | Response |
| --- | --- | --- | --- | --- |
| GET | /api/metrics/history | JWT | durationMinutes=int (default 60) | [{"distance":int,"safety":int,"mode":int,"solarVoltage":float,"timestamp":"datetime"}] (~500 pts max) |
| GET | /api/metrics/stats | JWT | durationMinutes=int (default 60) | {"totalCount":int,"averageSolarVoltage":float,"maxDistance":int,"minDistance":int,"averageDistance":float,"safetyIncidentCount":int,"modeDistribution":{"0":int,"1":int,"2":int,"3":int}} |
| DELETE | /api/metrics/all | JWT+Admin | — | {"message":"Cleared N records"} |